DEPARTMENT OF COMPUTER & SOFTWARE ENGINEERING

COLLEGE OF E&ME, NUST, RAWALPINDI

Subject Name

Digital Image Processing

Lab Number

4

SUBMITTED TO:

Dr. Asad Khan

LE Sundas Ashraf

SUBMITTED BY:

Student Name

- Wahaaj Nasir

Reg#413238

DE- 44 Dept C&SE

Objectives:

Filtering on Images and Histogram Calculations

Related Topic/Chapter in theory class:

Spatial Filtering

Hardware/Software required:

Hardware: PC

Software Tool: Pycharm

Task 1:

Apply contrast stretching on the image provided by setting 5th and 95th percentiles of the input values to 0 and 255 respectively. The remaining values will be stretched as: N𝑒𝑤 𝑉𝑎𝑙𝑢𝑒 𝑓𝑜𝑟 𝑃𝑖𝑥𝑒𝑙 = 255 ∗ (𝐶𝑢𝑟𝑟𝑒𝑛𝑡 𝑣𝑎𝑙𝑢𝑒 𝑓𝑜𝑟 𝑃𝑖𝑥𝑒𝑙 − 5𝑡ℎ 𝑃𝑒𝑟𝑐𝑒𝑛𝑡𝑖𝑙𝑒) / (95𝑡ℎ 𝑃𝑒𝑟𝑐𝑒𝑛𝑡𝑖𝑙𝑒 − 5𝑡ℎ 𝑃𝑒𝑟𝑐𝑒𝑛𝑡𝑖𝑙𝑒)

Solution:

import cv2 as cv
import numpy as np

def contrast_stretch(image):
    im_min_5 = np.percentile(image, 5)
    im_max_95 = np.percentile(image, 95)
    rows,cols = image.shape
    new_img = np.zeros((rows, cols), dtype = np.uint8)

    for i in range(rows):
        for j in range(cols):
            if(image[i][j] < im_min_5):
                new_img[i][j] = 0
            elif(image[i][j] > im_max_95):
                new_img[i][j] = 255
            else:
                new_img[i][j] = 255 * ((image[i][j] - im_min_5) / (im_max_95 - im_min_5))

    return new_img


#---------------------------------
# Main

image = cv.imread("D:/Uni/Semester 6/DIP/Self/Lab/Lab 4/Lab 4/low_con.jpg", 0)
cont_img = contrast_stretch(image)

cv.imshow('Original Image', image)
cv.imshow('Contrast Image', cont_img)
cv.waitKey()

Output:

Task 2:

Perform the following steps to do Hist. equalization to enhance the grayscale image.

- Calculate the histogram of the image and display it using the appropriate command.

(Don’t use the built in function of OpenCV or Numpy or Matplotlib etc.)

- Calculate probability density function (PDF) from the histogram and display it using the appropriate command PDF = H/(R*C). Where H is the Histogram and R and C is the number of Rows and Columns of the image respectively.

- Calculate cumulative PDF and display it using the appropriate command.

- Multiply the Cumulative PDF with 255 to find the transformation function then display it too using the appropriate command.

- From the transformation function, replace the gray levels of the image to create contrast enhanced (histogram equalized) image.

- Display the enhanced image.

Solution

import cv2 as cv
import numpy as np
import matplotlib.pyplot as plt

def histogram_creating(image):
    rows, cols = image.shape
    histogram = np.zeros(256, dtype = int)

    for i in range(rows):
        for j in range(cols):
            val = image[i][j]
            histogram[val] += 1

    return histogram

def calc_pdf(histogram, image):
    rows, cols = image.shape

    histogram = histogram/(rows*cols)

    return histogram

def calc_cum_pdf(pdf):
    cum_pdf = np.zeros(len(pdf), dtype = float)
    cum_pdf[0] = pdf[0]
    for i in range(1, len(pdf)):
        cum_pdf[i] = cum_pdf[i-1] + pdf[i]

    return cum_pdf

def transformation_fun(cum_pdf):
    return np.uint8((cum_pdf*255))

def apply_trans(image, trans_fun):
    rows, cols = image.shape
    new_img = np.zeros((rows, cols), dtype = np.uint8)

    for i in range(rows):
        for j in range(cols):
            new_img[i][j] = trans_fun[image[i][j]]

    return new_img

# ---------------------------------
# Main
image = cv.imread("D:/Uni/Semester 6/DIP/Self/Lab/Lab 4/Lab 4/fig05.tif", 0)
cv.imshow('Original Image', image)
cv.waitKey()
histogram = histogram_creating(image)

plt.plot(histogram)
plt.xlabel('Pixel Indices')
plt.ylabel('Pixel Count')
plt.title('Histogram')
plt.show()

pdf = calc_pdf(histogram, image)
plt.plot(pdf)
plt.xlabel('Pixel Indices')
plt.ylabel('Pixel Intensity')
plt.title('Probability Density Function')
plt.show()

cum_pdf = calc_cum_pdf(pdf)
plt.plot(cum_pdf)
plt.xlabel('Pixel Indices')
plt.ylabel('Pixel Intensity')
plt.title('Cumulative PDF')
plt.show()

transformation_function = transformation_fun(cum_pdf)
plt.plot(transformation_function)
plt.xlabel('Pixel Indices')
plt.ylabel('Pixel Count')
plt.title('Transformation Function')
plt.show()

print(transformation_function)

equilized_img = apply_trans(image, transformation_function)
cv.imshow('Enhanced Image', equilized_img)
cv.waitKey()

Output:

Original Image

Histogram

Probability Density Function

Cumulative PDF

Transformation Function

Low Contrast Image

Task 3:

Write a generic function that takes a mask of a specific size from and applies it to an image.

Create the following masks and apply it to an image and display the results. The fiter will

nxn size. You can use fig05.tif for this task.

Solution

import cv2 as cv
import numpy as np

def padding(pad, orig):
    rows, cols = orig.shape
    padded_arr = np.ones((rows+ 2 * pad, cols+ 2 * pad), dtype = np.uint8)*255

    for i in range(rows):
        for j in range(cols):
            padded_arr[i+pad][j+pad] = orig[i][j]

    return padded_arr

def remove_padding(padded_img, pad):
    rows, cols = padded_img.shape
    return padded_img[pad:rows-pad, pad:cols-pad]

def filter(image, filter_size, filter_var):
    pad = filter_size//2
    rows, cols = image.shape
    filtered_img = np.zeros((rows, cols), dtype = np.uint8)

    padded_img = padding(pad, filtered_img)
    for i in range(pad, rows-pad):
        for j in range(pad, cols-pad):
            sub_img = image[i-pad:i+pad+1, j-pad:j+pad+1]
            sub_img = np.multiply(sub_img, filter_var)
            sub_img_val = np.sum(sub_img)

            padded_img[i][j] = sub_img_val

    filtered_img = remove_padding(padded_img, pad)

    return filtered_img

# ----------------------------------
# Main
image = cv.imread("D:/Uni/Semester 6/DIP/Self/Lab/Lab 4/Lab 4/fig05.tif", 0)
cv.imshow('Original Image', image)
cv.waitKey()

filtered = filter(image, 3, 1/9)
cv.imshow('Filtered Image', filtered)
cv.waitKey()

Output:

Original Image

Log Transform